SIMATS ENGINEERING

ASSESSMENT TOOL 2

# COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07  	 	 	NAME’Jayashree M

REG NO;192524373

# COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards  (BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile devices.

The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisychained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8- or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiberbased ring or mesh at the campus backbone level for redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industryrelevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem- Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and welldocumented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |

ANSWERS

3. Bus Topology for a Small Classroom Lab

For a small classroom computer lab with a limited budget, a bus topology is one of the simplest network topologies to explain and implement. In a traditional bus topology, all computers are connected to one common communication cable called the backbone. Every computer shares this communication medium for transmitting and receiving data.

The major advantage is the low hardware requirement and simple physical structure. However, traditional coaxial bus networks are largely obsolete today and have been replaced by switched Ethernet networks. Therefore, for an academic answer, it is useful to explain both the traditional bus topology and the modern practical alternative.

- What is Bus Topology?

A bus topology is a network arrangement in which all computers or network devices are connected to a single main cable called the backbone cable.

The backbone acts as a common communication path. When one computer transmits data, the signal travels along the backbone and can be received by the other computers connected to it.

Basic representation

BUS / BACKBONE CABLE

─────────────────────────────────────────────

│          │          │          │

│          │          │          │

PC-1       PC-2       PC-3       PC-4

The backbone has two ends, and in a traditional coaxial Ethernet network, each end must be terminated properly to prevent signal reflections.

- Why Bus Topology is Suitable for a Low-Budget Classroom

Suppose a classroom has 10–15 computers and the main requirement is basic:

- Internet browsing

- File sharing

- Printing

- Accessing educational resources

- Sharing a teacher's computer

- Basic LAN communication

A traditional bus topology can reduce the number of networking devices because computers share one backbone.

Main cost-saving features

- Single backbone cable

- No central switch in the traditional design

- Fewer networking devices

- Simple physical installation

- No dedicated server required

- Low cable requirement compared with some other topologies

- Easy to understand and maintain for a small experimental laboratory

3. Traditional Bus Topology Diagram

A proper traditional coaxial Ethernet bus network can be represented as follows:

TRADITIONAL BUS TOPOLOGY

50 Ω Terminator                    50 Ω Terminator

│                                  │

▼                                  ▼

┌──────────┐──────────────────────────────────┐

│          │                                  │

│          │                                  │

└──────────┘                                  │

│                                        │

│                                        │

┌─────┐                                  ┌─────┐

│ T   │                                  │ T   │

└──┬──┘                                  └──┬──┘

│                                        │

PC-1                                     PC-4

│

┌────┴────┐

│  BNC    │

│Connector│

└─────────┘

│

│

────────┼────────────────────────────────────────

│

┌───┐

│ T │

└─┬─┘

│

PC-2

│

┌────┴────┐

│    T    │

└────┬────┘

│

PC-3

A simpler representation is:

50Ω Terminator                                      50Ω Terminator

│                                                     │

▼                                                     ▼

├───────────────┬──────────────┬──────────────┬───────┤

│              │              │

T-Connector    T-Connector    T-Connector

│              │              │

PC-1           PC-2           PC-3

- Components Required

For a traditional coaxial bus network, the following components are required.

| Component | Purpose |
| --- | --- |
| Computers/PCs | Network nodes |
| Coaxial cable | Main communication backbone |
| RG-58 cable | Common 50 Ω coaxial cable for classic Ethernet |
| BNC connectors | Connect coaxial cable to network interface |
| BNC T-connectors | Connect each computer to the backbone |
| 50 Ω terminators | Terminate both ends of the backbone |
| Network Interface Cards | Provide network connectivity |
| Ethernet adapters | Connect PCs to the network |
| Patch cables | Short connections where required |
| RJ-45 cables | Used in modern Ethernet alternatives |
| Switch | Used in modern switched Ethernet |
| Router | Provides Internet connectivity |
| Power supply | Powers active network equipment |

- RG-58 Coaxial Cable

The traditional Ethernet bus network uses coaxial cable, commonly RG-58, with an approximate characteristic impedance of 50 Ω.

The cable contains:

Outer Jacket

┌─────────────────┐

│   Shield        │

│ ┌─────────────┐ │

│ │ Insulation  │ │

│ │   ┌─────┐   │ │

│ │   │Core │   │ │

│ │   └─────┘   │ │

│ └─────────────┘ │

└─────────────────┘

The central conductor carries the electrical signal while the shielding helps protect the signal from electromagnetic interference.

- BNC T-Connector

Each computer is connected to the backbone using a BNC T-connector.

Conceptually:

Backbone

────────────────┬────────────────

│

┌──┴──┐

│ BNC │

│  T  │

└──┬──┘

│

│

PC

The T-connector allows the main cable to continue in both directions while providing a connection to the computer's network interface.

7. BNC Connector

A BNC connector is a bayonet-style connector used to connect coaxial cable to compatible networking equipment.

Simplified structure:

Coaxial Cable

───────────────[ BNC ]──────────── Network Interface

It provides a secure connection and was commonly used with early Ethernet coaxial networks.

- 50-Ohm Terminators

One of the most important components in a traditional bus network is the 50 Ω terminator.

There must be a terminator at both physical ends of the backbone.

50 Ω Terminator                                      50 Ω Terminator

│                                                    │

▼                                                    ▼

┌───────┐                                            ┌───────┐

│  50Ω  │════════════════════════════════════════════│  50Ω  │

└───────┘        Backbone Cable                     └───────┘

Why are terminators necessary?

When an electrical signal reaches the end of an unterminated cable, part of the signal can be reflected back.

This can cause:

- Signal distortion

- Data corruption

- Communication errors

- Network instability

The terminator absorbs the signal energy and reduces reflections.

Therefore:

Both ends of a traditional coaxial bus must be terminated with the appropriate 50 Ω termination.

9. How Data Transmission Works

Consider four computers:

PC1 ───── PC2 ───── PC3 ───── PC4

Common Backbone

Suppose PC1 wants to send data to PC3.

The process is approximately:

PC1

│

│ Data transmission

▼

══════════════════════════════════════

Backbone

══════════════════════════════════════

│                  │

▼                  ▼

PC2                PC3

│

▼

Receives data

The signal travels through the shared communication medium.

The destination computer examines the transmitted frame and accepts it if the destination address matches.

10. CSMA/CD and Collisions

Traditional shared Ethernet bus networks use CSMA/CD — Carrier Sense Multiple Access with Collision Detection.

The basic idea is:

Step 1 — Carrier Sense

A computer checks whether another device is currently transmitting.

Is the cable free?

│

┌───┴───┐

│       │

YES      NO

│       │

Transmit   Wait

Step 2 — Multiple Access

Multiple computers can share the same communication medium.

Step 3 — Collision Detection

If two computers transmit at approximately the same time, their signals can collide.

PC1 ───────────────►

X  COLLISION

PC2 ───────────────►

The computers detect the collision and wait for a randomly selected period before attempting retransmission.

11. Major Advantages of Bus Topology

1. Low Cost

The biggest advantage is the relatively small amount of networking hardware required in a traditional bus network.

2. Simple Structure

The network has a straightforward physical arrangement:

PC ──┬── PC ──┬── PC ──┬── PC

│         │         │

└──────── Backbone ─┘

3. Less Cable

A single backbone can pass along the classroom, with computers attached at different points.

4. No Central Hub Required

Traditional bus Ethernet does not require a central hub or switch.

5. Easy to Understand

It is useful for teaching:

- LAN concepts

- Ethernet

- Shared media

- CSMA/CD

- Network topology

- Signal termination

6. Suitable for Small Networks

For a small number of computers with light network traffic, the design can be adequate in a laboratory demonstration.

7. No Dedicated Server Required

A dedicated server is not mandatory.

For example, a teacher's computer can provide shared resources using:

- Windows file sharing

- Linux Samba

- Shared folders

- Network printers

12. Disadvantages of Bus Topology

Despite its low cost, bus topology has several significant disadvantages.

1. Single Point of Failure

The biggest problem is the backbone.

PC1 ─── PC2 ─── X ─── PC3 ─── PC4

↑

Cable break

A major break in the backbone can prevent communication across the network.

2. Collisions

Because multiple computers share the same medium, simultaneous transmissions can cause collisions.

3. Performance Decreases with More Devices

As the number of computers and network traffic increases, the shared medium becomes increasingly congested.

4. Difficult Fault Detection

Finding the exact location of a cable or connector problem can be difficult.

5. Limited Scalability

Bus topology is not ideal for large networks.

6. Terminator Problems

If one terminator is missing or improperly connected, network communication can become unreliable.

7. Cable Damage Affects Multiple Devices

A single backbone problem can affect many or all computers.

13. Classroom Lab Layout

Suppose there are 10 computers arranged along two rows of desks.

A traditional conceptual arrangement could be:

CLASSROOM BUS NETWORK

50Ω                                                        50Ω

Terminator                                                 Terminator

│                                                           │

▼                                                           ▼

├───────────┬───────────┬───────────┬───────────┬──────────┤

│           │           │           │

PC1         PC2         PC3         PC4

│

│

PC5

│

│

PC6

├───────────┬───────────┬───────────┬───────────┬──────────┤

│           │           │           │

PC7         PC8         PC9         PC10

The actual physical arrangement would depend on the classroom's desk layout and cable-routing requirements.

14. Internet Connection

A traditional bus network itself does not automatically provide Internet access.

To provide Internet access, a router or gateway is required.

Conceptually:

INTERNET

│

│

┌───────┐

│ Router│

└───┬───┘

│

│

Network connection

│

▼

═══════════════════════════════

BUS BACKBONE

═══════════════════════════════

│       │       │       │

PC1     PC2     PC3     PC4

The router provides connectivity between the classroom LAN and the Internet.

15. Resource Sharing Without a Dedicated Server

A dedicated server is not necessarily required for a small classroom.

For example, the teacher's computer can act as a resource-sharing computer.

Teacher PC

┌─────────────┐

│ Shared Files│

│ Documents   │

│ Resources   │

└──────┬──────┘

│

│

Shared Network

│

┌──────────────┼──────────────┐

│              │              │

PC1            PC2            PC3

Students can access shared folders if appropriate permissions are configured.

16. Modern Alternative — Switched Ethernet

There is an important practical point:

Traditional coaxial bus Ethernet is obsolete in modern computer labs.

Today, a classroom that needs low cost, reliability and easy maintenance would normally use Ethernet over Cat5e/Cat6 with an unmanaged switch.

However, this is technically a star topology, not a true bus topology.

Modern arrangement

Internet

│

Router

│

│

┌─────────────┐

│  8/16-Port  │

│   Switch    │

└─────────────┘

│ │ │ │ │ │

│ │ │ │ │ └──── PC6

│ │ │ │ └────── PC5

│ │ │ │

│ │ │ └──────── PC4

│ │ └────────── PC3

│ └──────────── PC2

└────────────── PC1

This uses Cat5e or Cat6 UTP cables and an inexpensive unmanaged Ethernet switch.

17. Important Correction About the Modern Variation

It is better not to describe an unmanaged switch as a bus topology.

A switch creates a physical star topology because every computer has an individual cable running to the switch.

PC1

│

│

PC2 ───────── Switch ───────── PC3

│

│

PC4

Although the switch-based network provides a similar advantage of simple centralized connectivity, it is technically classified as a star topology.

Therefore, in an examination answer, you can write:

"The traditional bus topology uses a shared coaxial backbone with T-connectors and 50 Ω terminators. In modern classroom networks, the practical low-cost replacement is switched Ethernet using an unmanaged switch and Cat5e/Cat6 cables; however, this is physically a star topology rather than a true bus topology."

This distinction can earn extra marks because it demonstrates technical accuracy.

18. Traditional Bus vs Modern Switched Ethernet

| Feature | Traditional Bus | Modern Switched Ethernet |
| --- | --- | --- |
| Main medium | Coaxial cable | Cat5e/Cat6 UTP |
| Topology | Bus | Star |
| Central device | Not required | Switch |
| Connector | BNC | RJ-45 |
| Terminators | 50 Ω required | Not required |
| Collisions | Possible | Normally avoided in full-duplex switching |
| Fault isolation | Difficult | Easier |
| Scalability | Poor | Good |
| Maintenance | Difficult | Easier |
| Current practical use | Obsolete/legacy | Widely used |
| Cost | Historically low | Very affordable today |
| Suitable for modern lab | Mainly educational | Yes |

19. Estimated Hardware for a Small Classroom

For a 10-PC modern classroom network, a practical low-cost setup could contain:

10 × Computers

1  × 16-port Gigabit unmanaged switch

10 × Cat6/Cat5e Ethernet cables

1  × Router

1  × Internet connection

1  × UPS/surge protection for network equipment

The switch provides enough ports for the PCs and leaves a few ports available for expansion.

Example

INTERNET

│

▼

┌───────┐

│Router │

└───┬───┘

│

▼

┌─────────────────┐

│ 16-Port Switch  │

└─────────────────┘

│ │ │ │ │ │ │ │

│ │ │ │ │ │ │ └── PC8

│ │ │ │ │ │ └──── PC7

│ │ │ │ │ └────── PC6

│ │ │ │ └──────── PC5

│ │ │ └────────── PC4

│ │ └──────────── PC3

│ └────────────── PC2

└──────────────── PC1

20. Installation Procedure — Traditional Bus

Step 1: Plan the classroom

Determine:

- Number of computers

- Location of each computer

- Cable path

- Location of the backbone

- Location of both cable ends

Step 2: Install the backbone

Run the RG-58 coaxial cable along the classroom.

Wall

──────────────────────────────────

Backbone cable

══════════════════════════════════

Step 3: Install T-connectors

At every computer location, install the appropriate BNC T-connector.

Backbone ────────┬────────

│

T

│

PC

Step 4: Connect computers

Connect each computer's network interface to its T-connector.

Step 5: Install terminators

Place a 50 Ω terminator at each physical end.

50Ω

│

▼

══════════════════════════════════

▲      ▲       ▲       ▲

│      │       │       │

PC1    PC2     PC3     PC4

══════════════════════════════════

▲

│

50Ω

Step 6: Configure network settings

Configure:

- IP address

- Subnet mask

- Gateway

- DNS

For example:

PC1 → 192.168.1.11

PC2 → 192.168.1.12

PC3 → 192.168.1.13

PC4 → 192.168.1.14

Subnet Mask → 255.255.255.0

Step 7: Test connectivity

Use:

ping 192.168.1.12

to verify communication between computers.

21. Troubleshooting

Problem 1: Entire network is down

Possible causes:

- Backbone cable disconnected

- Cable damaged

- Missing terminator

- Incorrect termination

- Faulty network adapter

Problem 2: One computer cannot communicate

Check:

- BNC connector

- T-connector

- Network adapter

- Cable connection

- IP configuration

Problem 3: Intermittent communication

Possible causes:

- Loose connectors

- Damaged coaxial cable

- Incorrect termination

- Electrical interference

Problem 4: Network becomes slow

Possible causes:

- Too many computers

- Heavy network traffic

- Excessive collisions

- Network hardware limitations

22. Advantages and Disadvantages — Exam Table

| Advantages | Disadvantages |
| --- | --- |
| Low hardware cost | Backbone failure can affect entire network |
| Simple physical design | Collision possibility |
| Requires less cable | Performance decreases with traffic |
| No central device in traditional design | Troubleshooting can be difficult |
| Easy to understand | Limited scalability |
| Suitable for small networks | Terminators are required |
| No dedicated server required | Legacy technology |
| Simple resource sharing | Difficult to expand |

23. Complete Architecture

A complete conceptual classroom network can be shown as:

┌───────────────────┐

│     INTERNET      │

└─────────┬─────────┘

│

▼

┌─────────────┐

│   ROUTER    │

└──────┬──────┘

│

│

TRADITIONAL BUS NETWORK

│

50Ω                  │                    50Ω

Terminator              │                 Terminator

│                    │                    │

▼                    ▼                    ▼

├────────────────────────────────────────┤

BACKBONE

├──────────┬──────────┬──────────┬────────┤

│          │          │

T          T          T

│          │          │

PC1        PC2        PC3

│

T

│

PC4

Teacher PC ── Shared Files

Student PCs ── Internet

└── File Sharing

└── Printer Sharing

24. Conclusion

For a small classroom laboratory with a very tight budget, the traditional bus topology provides a simple and economical networking concept because all computers share a common backbone. A traditional implementation uses 50 Ω coaxial cable such as RG-58, BNC connectors, BNC T-connectors, and 50 Ω terminators at both ends. It does not require a central switch or dedicated server.

Its major disadvantages are collisions, difficult troubleshooting, poor scalability, and the possibility that a backbone failure can disrupt the entire network.

For an actual modern classroom installation, however, the recommended approach is Cat5e/Cat6 Ethernet with an inexpensive unmanaged switch. This is technically a star topology, but it is inexpensive, significantly more reliable, easier to troubleshoot, and much more practical than legacy coaxial bus Ethernet.

Key points to remember for an exam

Bus topology → One common backbone → T-connectors → BNC connectors → 50 Ω terminators → Shared medium → CSMA/CD → Collisions possible → Backbone failure affects network → Low cost but poor scalability.